## Experiment No.: 07	Date:	/	/

Half Adder and Full Adder

AIM:	Write VHDL code for Half adder and Full Adder using data flow modeling style, synthesis, simulate it and verify its functionality.

# EQUIPMENTS & COMPONENTS:

1) Computer with Xilinx Software / Modelsim Student Edition

# THEORY:

The half adder adds two one-bit binary numbers A and B. It has two outputs, Sum and Carry.

## Truth Table and Logical diagram of 1-Bit Half Adder:

| 1-Bit Half adder |  |  |
| --- | --- | --- |
| A | B | Y |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Fig. Logical Diagram of 1-Bit Half adder Equations of Half adder:

Sum = A  XOR B Carry = A AND B

# Bit Full Adder:

A full adder adds binary numbers and accounts for values carried in as well as out. A one-bit full adder adds three one-bit numbers, often written as A, B, and Cin; A and B are the operands, and Cin is a bit carried in from the next less significant stage.

## Truth Table and Logical diagram of 1-Bit full Adder:

Fig. Logical Diagram of 1-Bit full adder Equations of 1-bit full adder:

Sum = A XOR B XOR C

Carry = (A AND B) OR (A AND C) OR (B AND C)

# PROCEDURE:

  - Write a VHDL code for Half adder/Full adder.

  - Check syntax for syntax errors if any.

  - Synthesize the code using XST tool and generate RTL schematic.

  - Simulate the VHDL Code of Half adder/Full adder using Isim Simulator of Xilinx.

CONCLUSIONS: